The Lunascan Project - MOONWATCH - Team Report 
September 30, 2011

| TARGET: | SECTION 26/37 | MARE CRISIUM |
| --- | --- | --- |

MISSION PLAN: Test LPS unit/Celestron 80GT with CMOS camera, WWV transmitter/receiver. Camera 1 mini finder switchable to CCD camera Camera 2 Internal Camera 3 LPS/CMOS/LPI / Skyglobe graphics Camera 4 HPS/CMOS/SSI / VMA graphics Date incorrect on Quad. S/b 2011 09 30 CAMERAS: #### CAMERA ONE POSITION ON QUAD * Finder: mini cam CAMERA TWO POSITION * Internal camera #### CAMERA THREE POSITION * LPS Unit OPTICS Celestron NexStar 80GT, 80 mm (3.1”) 400 mm f/l f/5 focal ratio; aperture video data recorder on Quad VHS1 #### DVR1 Sony only. #### Camera used is Meade LPI/CMOs. (Meade Lunar Planetary Imager / complementary metal oxide semiconductor) OPTICS Celestron, 8" 2032 mm/ prime focus Resolution 0.68 arc/sec LPI equiv 6 mm, (90x (w/Barlow 150x) VGA resolution (640x480) color CMOS chip 30 fps Switchable to SKYGLOBE graphics Digital to Analog Converter 2; TEP-100 Elite Pro II, aperture video VHS2 #### CAMERA FOUR POSITION * HPS Unit 3: CCD/SSI Camera, (Celestron Neximage Solar System Imager / Charge Coupled Device) OPTICS Celestron, 8" 2032 mm/ prime focus (SSI equiv 5 mm, w/Barlow 150x) VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 Digital to Analog Converter 1; TEP-100 Elite Pro, aperture video DVR2 Switchable to * VMA (Virtual Moon Atlas) graphics CONFIG Celestron C-8, no diagonal, hand control, battery eliminator on drive Resolution 0.68 arcsec / lunar target at 0.5 degree/2160 miles wide = 0.23 miles or 1214’. 19-23-30 Mare Crisium on SSI/CCD #### REPORT NOTES VHS1 Quad DVR1 DVR2 Date on Quad was incorrect. Times stamped at 2011 10/30 s/b 2011 09/30 LPI unit did not focus so was not used. Imaging on Camera 4 was good No WWV signal on any recordings. 19:45:00 (Q) Ray west of Crisium EOS at 01H 14M elapsed No anomalies seen or recorded. T