The Lunascan Project - MOONWATCH - Team Report 
May 11, 2012

| TARGETS: | Western Half | No Specific Targets This Scan |
| --- | --- | --- |

MISSION PLAN: Routine scans of terminator, targeting Western Section Camera 1 mini finder switchable to CCD camera Camera 2 Internal Camera 3 LPI/VMA graphics. (LPI not used) Camera 4: SSI/CCD/ C-8. . . CAMERAS: #### CAMERA ONE POSITION ON QUAD * Finder: mini cam CAMERA TWO POSITION * Internal camera #### CAMERA THREE POSITION Compaq Presario sr1218nx computer (1) Digital to Analog Converter 1; TEP-100 Elite Pro II Aperture video recorder VHS2 VIRTUAL MOON ATLAS graphics #### CAMERA FOUR POSITION Dell computer Digital to Analog Converter 2: TEP-100 Elite Pro II Aperture video recorder DVR2 No graphics used All camera outputs to Quad / VHS1 / Q Select DVR1 SSI Camera CCD, (Celestron Neximage Solar System Imager / Charge Coupled Device) OPTICS, prime focus. 5 mm VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 LPI Camera Not used CMOS, (Meade Lunar Planetary Imager / complementary metal oxide semiconductor) OPTICS, prime focus. 6 mm. VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 Celestron C-8 SCT 8” optical tube 2032 mm f/l F10 Resolution at lunar range = 0.68 / .81086 mi or 4281.36469 feet (30.19' X 60 = 1811.4" /.68' resol = factor of 2663.82353 2160 miles / 2663.82353 = .81086 mi resolution) 0.621 mi/km 1 m = 3.28 ‘ CONFIG No diagonal, extended hand control DOB Driver II #### REPORT NOTES #### 2032 mm / 5 mm = 400x . Lunar range 396,200 km = 242,935 mi / 400 = 600 mi sim range #### No audio on recordings by DVR 1 or 2 #### No finder cable. Need to go wireless. Take transm off QuadS and use on scope finder #### No tape in VCR1, no Quad recordings #### No WWV signal. Need better receiver, crystal controlled if possible. #### Bad picture on Qselect Mon for some reason #### Desktop on Mon 4 keeps shutting down but set at 120mins #### DVR1 #### Stop #### DVR2 #### No sound, good images, unredacted Recordings on but no sound VHS1 WWV Quad audio not recd and no tape in drive VHS2 Used for VMA but not saved / CMOS cam not used DVR1 no audio DVR2 no audio VGA computer display from Celestron CCD camera, very good. Recording from DVR analyzed No anomalies Cam 1 quadrant needs overlay for targeting. Problem with reception of WWV signal from Fort Collins still an issue. Scope Note needs to be added to op manual to - Balance on scope needs to be reviewed - F discs need to be in place before BOS - Finders (both) and scope need to be aligned before BOS. - Orientation of Moon needs to be performed before BOS. - Due to high angle of most shots and the new mount the binoc viewer and diagonal must be used. - Post mode sheet for DOB Driver II in cab / learn modes - Calibrate DOB Driver II for BLASH 1. Compaq Presario sr1218nx XP home, dvd rom, cd burner, 2.67 GHZ processor, 760 mb RAM, 80 GB hard drive T