The Lunascan Project - MOONWATCH - Team Report 
May 30, 2012

| TARGETS: | Eastern Limb | No Specific Targets This Scan |
| --- | --- | --- |

#### BOS MISSION PLAN: Routine scans with 73.4% illumination / phase 62.1 Lunar range at 363,579 km / 226.000 miles / lunation 10.09 days / colongitude 34.7 Simrange 600 miles Start Q at 21:53:30 Target acq at 22:01:10 due to obstruction Good imaging at 22:07:00 with good WWV Camera 1 mini finder Camera 2 Cargo cams (1 &2) Camera 3 LPI/VMA graphics. (LPI not used) Camera 4: SSI/CCD/ C-8. . . CAMERAS: #### CAMERA ONE POSITION ON QUAD * Finder: mini cam CAMERA TWO POSITION * Cargo cameras color minicam & night vision #### CAMERA THREE POSITION Compaq Presario sr1218nx computer (1) Digital to Analog Converter 1; TEP-100 Elite Pro II VIRTUAL MOON ATLAS graphics #### CAMERA FOUR POSITION Dell computer Digital to Analog Converter 2: TEP-100 Elite Pro II Aperture video recorder DVR2 No graphics used All camera outputs to Quad / VHS1 / Q Select DVR1 SSI Camera CCD, (Celestron Neximage Solar System Imager / Charge Coupled Device) OPTICS, prime focus. 5 mm VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 LPI Camera Not used CMOS, (Meade Lunar Planetary Imager / complementary metal oxide semiconductor) OPTICS, prime focus. 6 mm. VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 Celestron C-8 SCT 8” optical tube 2032 mm f/l F10 Resolution at lunar range = 0.68 / .81086 mi or 4281.36469 feet (30.19' X 60 = 1811.4" /.68' resol = factor of 2663.82353 2160 miles / 2663.82353 = .81086 mi resolution) 0.621 mi/km 1 m = 3.28 ‘ CONFIG No diagonal or binoc viewer. Extended hand control DOB Driver II #### REPORT NOTES #### Acq at 7m11s but not clear of obstruction #### Good WWV #### Time fix at 3H3m CUT = 10m4s elapsed #### Images improve 13m elapsed #### Clears trees #### Copernicus 14m #### Bird 42m32s #### No anomales #### 2032 mm / 5 mm = 400x . = 600 mi sim range #### BOS #### Sky conditions were excellent with some halo later in mission. Able to shoot the full two hours. #### WWV signal very good with plenty of time fixes. Signal improved many times with arm up!!! Quad within 1-2 secs of Fort Collins time signal. #### Craters at top right are Helicon & LeVerrier (Section 10 & 11). Below center is Mons La Hire (20) L-R bottom are Euler, Lambert (20) and Timocharis (21). #### . #### Blue instrument lights and color mini cargo cam were tested. #### Aperture video VHS3 used on Cargo cams 1. Compaq Presario sr1218nx XP home, dvd rom, cd burner, 2.67 GHZ processor, 760 mb RAM, 80 GB hard drive T