The Lunascan Project - MOONWATCH - Team Report 
June 13, 2012

| TARGETS: Multi | Eastern Limb | Sunset on Copernicus |
| --- | --- | --- |

#### BOS 04:00:00 Quad #### Acq of signal at 4:02:51 Quad due to obstruction in East #### Time fix at 4:03:01 Quad = 9h03m CUT = 2m47s elapsed #### EOS at 5:09:45 Quad MISSION PLAN: Routine scans with 73.4% illumination / phase 62.1 Lunar range at 363,579 km / 226.000 miles / lunation 10.09 days / colongitude 34.7 Simrange 600 miles Good imaging with good WWV Camera 1 mini finder Camera 2 Cargo cams (1 &2) Camera 3 LPI/VMA graphics. (LPI not used) Camera 4: SSI/CCD/ C-8. . . CAMERAS: #### CAMERA ONE POSITION ON QUAD * Finder: mini cam CAMERA TWO POSITION * Cargo cameras color minicam & night vision #### CAMERA THREE POSITION Compaq Presario sr1218nx computer (1) Digital to Analog Converter 1; TEP-100 Elite Pro II VIRTUAL MOON ATLAS graphics #### CAMERA FOUR POSITION Dell computer Digital to Analog Converter 2: TEP-100 Elite Pro II Aperture video recorder DVR2 No graphics used All camera outputs to Quad / VHS1 / Q Select DVR1 SSI Camera CCD, (Celestron Neximage Solar System Imager / Charge Coupled Device) OPTICS, prime focus. 5 mm VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 LPI Camera Not used CMOS, (Meade Lunar Planetary Imager / complementary metal oxide semiconductor) OPTICS, prime focus. 6 mm. VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 Celestron C-8 SCT 8” optical tube 2032 mm f/l F10 Resolution at lunar range = 0.68 / .81086 mi or 4281.36469 feet (30.19' X 60 = 1811.4" /.68' resol = factor of 2663.82353 2160 miles / 2663.82353 = .81086 mi resolution) 0.621 mi/km 1 m = 3.28 ‘ CONFIG No diagonal or binoc viewer. Extended hand control DOB Driver II #### REPORT NOTES #### BOS 4:00:00 Quad due to obstruction to the East #### Good A/V with WWVR near Moonbuggy, no ant #### Good recordings except for late start with DVR1. Apparent malfunction on startup. Also no audio on recording. #### Good A/V on VCR1,3, DVR2 #### Time fix at 8H3m CUT = 1:02:12 Elapsed = 5:03 AM #### Cap 061 #### EOS elapsed 1:09:25 #### No anomalies observed. #### SUGGESTIONS #### Position marker on Quad cam1 pos #### Check out DVR1 for startup and audio #### Shorten line to cargo cam VCR3 and use line on Findercam #### 2032 mm / 5 mm = 400x . = 600 mi sim range 1. Compaq Presario sr1218nx XP home, dvd rom, cd burner, 2.67 GHZ processor, 760 mb RAM, 80 GB hard drive T