The Lunascan Project - MOONWATCH - Team Report 
September 18, 2011

| TARGET: | SECTION 31 | COPERNICUS |
| --- | --- | --- |

MISSION PLAN: Camera 1 mini finder switchable to CCD camera Camera 2 Internal Camera 3 CMOS/LPI / Skyglobe graphics Camera 4 CMOS/SSI / VMA graphics CAMERAS: On video switching #### CAMERA ONE POSITION ON QUAD * Finder: mini cam * LPS Unit (NIS this date): CCD, GBC-200 (charge coupled device) OPTICS Celestron NexStar 80GT, 80 mm (3.1”) 400 mm f/l f/5 focal ratio; aperture video data recorder on Quad VHS1 DVR1 Sony only. Camera used is optional but planned as GBC-200 with regular T-C adaptor and 25mm eyepiece. #### CAMERA TWO POSITION * Internal camera when HPS 1 not in use * HPS Unit 1: CCD, GBC-400 (charge coupled device) OPTICS Celestron, 8" 2032 mm / 26 mm plossyl in adjustable T-C adaptor; aperture video data recorder on Quad VHS1 DVR1 Sony #### CAMERA THREE POSITION * HPS Unit 2: CMOS/LPI (Meade Lunar Planetary Imager / complementary metal oxide semiconductor) OPTICS Celestron, 80 mm x 400 mm f/l 80GT / prime focus equiv 6mm (LPI= 80x) or 5 mm (SSI= 66x) OR Celestron, 8" 2032 mm/ prime focus equiv 6 mm, (90x (w/Barlow 150x) VGA resolution (640x480) color CMOS chip 30 fps Switchable to SKYGLOBE graphics Digital to Analog Converter 2; TEP-100 Elite Pro II, aperture video VHS2 #### CAMERA FOUR POSITION * HPS Unit 3: CMOS/SSI Camera, (Celestron Neximage Solar System Imager / complementary metal oxide semiconductor) OPTICS Celestron, 8" 2032 mm/ prime focus (SSI equiv 5 mm, w/Barlow 150x) VGA resolution (640x480) color ¼” CMOS chip 30 fps Compression 1420 Digital to Analog Converter 1; TEP-100 Elite Pro, aperture video DVR2 Switchable to * VMA (Virtual Moon Atlas) graphics CONFIG Celestron C-8, no diagonal, hand control, battery eliminator on drive Resolution 0.68 arcsec / lunar target at 0.5 degree/2160 miles wide = 0.23 miles or 1214’. #### REPORT NOTES Section 31 Copernicus updated T